MQTT API Documentation

Fuel Level Monitor — SOJI Sensor

For Web & Backend Developers

This document describes the MQTT communication protocol between the ESP32 fuel monitoring device and the web application. It covers all topics, payload schemas, configuration options, and includes a JavaScript quick-start example.

| Device | ESP32 + SIM7600 modem + SOJI sensor |
| --- | --- |
| Protocol | MQTT v3.1.1 |
| Broker (default) | broker.hivemq.com |
| Document scope | Topics, payloads, config, JS example |

# Table of Contents

Table of Contents	2

1.  Overview	3

Responsibilities	3

2.  Connection Details	3

3.  Topic Structure	3

Full Topic List	4

4.  Telemetry Payload  (Device → Server)	5

JSON Payload Example	5

Field Reference	5

Signal Strength Guide	6

Level Calculation Notes	6

5.  Configuration Update  (Server → Device)	7

JSON Payload Example	7

Field Reference	7

Partial Updates	7

6.  Log Messages  (Device → Server)	8

JSON Payload Example	8

Field Reference	8

Common Error Codes	8

8.  Tank Shape Configuration Reference	9

Multi-Section Tank Example	9

9.  Message Flow Diagrams	10

Normal Telemetry Flow	10

Remote Configuration Flow	10

Offline Fallback and Retry	10

10.  Offline Behavior & Retry	10

# 1.  Overview

The device is an ESP32 board paired with a SOJI ultrasonic sensor that measures fuel level inside a tank. It communicates over a cellular network (SIM7600 modem) to an MQTT broker, then your web application subscribes to receive real-time data.

## Responsibilities

| Side | Publishes to | Subscribes to |
| --- | --- | --- |
| Device (ESP32) | telemetry, logs | config |
| Web App / Backend | config | telemetry, logs |

# 2.  Connection Details

| Parameter | Value |
| --- | --- |
| Broker | broker.hivemq.com |
| Port | 1883 (plain TCP) |
| Protocol | MQTT v3.1.1 |
| QoS (device pub.) | 1 (at-least-once) |
| Keep-alive | 60 seconds |
| Clean session | true |
| Authentication | None (public broker) |
| Will topic | device/mac/status |
| Will message | "offline" |

Production note: The broker and credentials are configurable in firmware. Replace broker.hivemq.com with your private broker before deploying to production.

# 3.  Topic Structure

All topics follow this pattern:

device/{PROJECT_ID}/{MAC_ADDRESS}/{channel}

| Placeholder | Description | Example |
| --- | --- | --- |
| {MAC_ADDRESS} | Device MAC address (unique per unit) | A4:CF:12:7E:3B:01 |
| {channel} | Message type | telemetry, config, logs |

## Full Topic List

| Channel | Direction | Full Topic Example |
| --- | --- | --- |
| telemetry | Device → Server | device/A4:CF:12:7E:3B:01/telemetry |
| config | Server → Device | device/A4:CF:12:7E:3B:01/config |
| logs | Device → Server | device/A4:CF:12:7E:3B:01/logs |

The telemetry and logs topics include the PROJECT_ID segment. The config and ota topics use a shorter path without it. To subscribe to ALL devices in the project at once use the MQTT wildcard: device/699700a5d383e0a593047e03/+/telemetry

# 4.  Telemetry Payload  (Device → Server)

Publish interval: every 30 seconds by default (configurable via the config topic). The device only publishes if at least one of level_cm, temp_c, or rssi has changed since the last send.

## JSON Payload Example

{

"level_pct": 45.2,

"level_cm": 90.4,

"volume_l": 180.8,

"capacity_l": 400.0,

"temp_c": 25,

"battery_mv": 3850,

"rssi": -85,

"firmware_version": "v1.0.0",

"hardware_version": "v1.0",

"ts": 1702500123,

"gps": {

"lat": 40.7128,

"lng": -74.0060,

"alt": 10.5,

"accuracy": 0.0

}

}

## Field Reference

| Field | Type | Unit | Description |
| --- | --- | --- | --- |
| level_pct | float | % | Fuel level as % of tank capacity (0–100) |
| level_cm | float | cm | Absolute fuel height in the tank |
| volume_l | float | liters | Calculated fuel volume based on tank geometry |
| capacity_l | float | liters | Total tank capacity |
| temp_c | int | °C | Fuel temperature measured by the sensor |
| battery_mv | int | mV | Device battery voltage |
| Rssi | int | dBm | Cellular signal strength (negative; closer to 0 = better) |
| firmware_version | string | — | Firmware version string |
| hardware_version | string | — | Hardware version string |
| Ts | int | Unix epoch (seconds) | Timestamp of the measurement (UTC) |
| gps.lat | float | degrees | Latitude (WGS84) |
| gps.lng | float | degrees | Longitude (WGS84) |
| gps.alt | float | meters | Altitude above sea level |
| gps.accuracy | float | meters | GPS accuracy estimate (0.0 if unavailable) |

## Signal Strength Guide

| rssi range | Quality |
| --- | --- |
| > −70 dBm | Excellent |
| −70 to −85 dBm | Good |
| −85 to −100 dBm | Fair |
| < −100 dBm | Poor |

## Level Calculation Notes

The sensor returns a raw 16-bit value (0–65535). The device converts it:

level_pct = (raw / 65535) × 100

level_cm  = (raw / 65535) × max_tank_height_cm

volume_l  = calculated using configured tank geometry

# 5.  Configuration Update  (Server → Device)

Send a JSON message to the device’s config topic to update its settings. The device applies the configuration immediately and persists it to flash memory (survives reboot).

Topic: device/{MAC_ADDRESS}/config

## JSON Payload Example

{

"reporting_interval_s": 30,

"timezone_offset_min": -300,

"gps_enabled": true,

"debug_mode": false,

"tank_shape": "rectangular",

"tank_shape_params": {

"length_m": 5.0,

"width_m": 3.0,

"height_m": 2.0,

"radius_m": 0.0,

"radius_b_m": 0.0

},

"tank_sections": []

}

## Field Reference

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| reporting_interval_s | int | 30 | How often the device publishes telemetry (seconds) |
| timezone_offset_min | int | 0 | UTC offset in minutes (e.g. UTC-5 = -300) |
| gps_enabled | bool | true | Enable/disable GPS readings |
| debug_mode | bool | false | When true, device publishes verbose logs to the logs topic |
| tank_shape | string | "rectangular" | Tank geometry type (see Section 8) |
| tank_shape_params | object | — | Dimensions for the selected shape |
| tank_sections | array | [] | Used for multi-section tanks only |

## Partial Updates

You can send only the fields you want to change. Omitted fields retain their current values:

{ "reporting_interval_s": 60 }

# 6.  Log Messages  (Device → Server)

When an error occurs or debug_mode is true, the device publishes log entries.

Topic: device/{MAC_ADDRESS}/logs

## JSON Payload Example

{

"level": "Failed",

"code": "SENSOR_CRC_ERROR",

"message": "Checksum mismatch: received 0x05, calculated 0x03",

"ts": 1702500123

}

## Field Reference

| Field | Type | Description |
| --- | --- | --- |
| Level | string | Severity: "Info", "Warning", "Failed" |
| Code | string | Machine-readable error code |
| Message | string | Human-readable description |
| Ts | int | Unix timestamp (UTC) |

## Common Error Codes

| Code | Meaning |
| --- | --- |
| SENSOR_CRC_ERROR | Sensor response failed CRC8 validation |
| SENSOR_TIMEOUT | No response from SOJI sensor |
| MQTT_PUBLISH_FAIL | Failed to publish; payload saved to SD card |
| MQTT_CONNECT_FAIL | Could not connect to MQTT broker |
| INVALID_JSON | Received malformed JSON on config/ota topic |
| SD_SAVE_FAIL | Could not write failed payload to SD card |

# 8.  Tank Shape Configuration Reference

The tank_shape string in the config payload selects the volume calculation algorithm.

| Value | Shape | Required params |
| --- | --- | --- |
| "rectangular" | Box | length_m, width_m, height_m |
| "cylinder_vertical" | Upright cylinder | radius_m, height_m |
| "cylinder_horizontal" | Lying cylinder | radius_m, length_m |
| "cone_vertical" | Vertical cone | radius_m, height_m |
| "ellipse_vertical" | Vertical ellipsoid | radius_m, radius_b_m, height_m |
| "sphere" | Sphere | radius_m |
| "capsule" | Cylinder + hemisph. caps | radius_m, height_m |
| "multi_section" | Compound shape | tank_sections array (max 8 sections) |

## Multi-Section Tank Example

Sections are stacked from bottom to top. Up to 8 sections are supported.

{

"tank_shape": "multi_section",

"tank_sections": [

{ "shape": "cylinder_vertical", "height_m": 1.0, "radius_m": 0.5 },

{ "shape": "rectangular",       "height_m": 0.5, "length_m": 1.0, "width_m": 1.0 }

]

}

# 9.  Message Flow Diagrams

## Normal Telemetry Flow

Device                      MQTT Broker                Web App

|                              |                          |

|-- publish telemetry -------->|                          |

|   (every 30s, QoS 1)         |-- deliver to subscriber->|

|                              |                          |

|<------------ PUBACK ---------|                          |

## Remote Configuration Flow

Web App                     MQTT Broker                Device

|                              |                          |

|-- publish config ----------->|                          |

|   device/{MAC}/config        |-- deliver -------------->|

|                              |                          |-- apply + save to flash

|                              |                          |-- (optional) log ack

## Offline Fallback and Retry

Device           SD Card           MQTT Broker

|                  |                   |

|  [broker down]   |                   |

|-- save JSON ----->|                   |

|                  |                   |

|  [broker up]     |                   |

|<-- read file -----|                   |

|-- publish --------------------------->|

|-- delete file --->|                   |

# 10.  Offline Behavior & Retry

When the cellular connection is unavailable:

- Failed telemetry payloads are saved to the device’s SD card as individual JSON files.

- The on-disk payload wraps the original telemetry with a timestamp and error code.

- When connectivity is restored, the device replays all saved payloads in order using the same telemetry topic.

- Successfully retried files are deleted from the SD card.

{

"timestamp": 1702500123,

"error_code": -1,

"payload": "{ ...original telemetry JSON... }"

}

Important for the web app: Retried messages may arrive out of chronological order relative to their ts field. Always use the ts field (not message arrival time) when ordering or displaying data.